I M.C.A II Semester Regular Examinations –May, 2024

COMPUTER NETWORKS

(MCA)

Time: 150 Min                                       	                             Max. Marks:80M

# SECTION – A

Answer all Four questions					                                      4×10M=40M

- a) Compare and contrast the OSI reference model with the TCP/IP reference model in terms

of their structure, layer organization, and historical development.                            (3M)

b) Define connection-oriented transmission in computer networks. What are the key characteristics that distinguish it from connectionless transmission?                            (4M)

c) Imagine a scenario where a large file needs to be transferred securely between two offices of a multinational company. The offices are connected through a dedicated, high-speed fiber-optic link. Discuss how a connection-oriented transmission approach could be beneficial in this situation.							   (3M)

- a) Discuss the objectives of flow control and the various mechanisms employed to achieve

it. 										               (3M)

b) Suppose a sender wants to transmit the message 1101101 using a simple parity check. Determine the parity bit (even) and the transmitted message.	                           (3M)

c) A bit stream 1001110 is transmitted using the standard CRC method. The generator polynomial is x3+1. Show the actual bit string transmitted. Suppose that the third bit from the left is inverted during transmission. Show that this error is detected at the receiver’s end. 									   (4M)

- a) Discuss the design issues of network layer and differentiate distance vector routing

algorithm and link state routing algorithms. 				               (4M)

b) Explain Classful IP addressing schemes in terms of address allocation, routing   efficiency, and scalability. 								   (3M)

c) A large number of consecutive IP address are available starting at 198.16.0.0. Suppose that four organizations, A, B, C, and D, request 4000, 2000, 4000, and 8000 addresses, respectively, and in that order. For each of these, give the first IP address assigned, the last IP address assigned, and the mask in the w.x.y.z/s notation.			   (3M)

- a) Why e-mail service is asynchronous communication and why not it will be synchronous

communication? 									   (2M)

b) What are the various application layer protocols involved in e-mail service? Discuss their usage in the e-mail system?					                           (4M)

c) Analyze the HTTP protocol used in the World Wide Web. Discuss its request-response nature, statelessness, and the role of methods such as GET, POST, PUT, and DELETE. 									                           (4M)

SECTION - B

Answer all Two questions 					                                     2×20M=40M

- a) Compare and contrast Stop-and-Wait and Sliding Window protocols in the context of

data link layer communication. 					                           (8M)

b) Consider a scenario where a sender and receiver are implanting the standard TCP congestion algorithm. The sender wants to send a 402 KB file to the receiver via an already established TCP connection. At the current time ‘t’ the following is the configuration of the hosts and the TCP connection:

- The ssthersh value at the sender is 64KB.

- Cwnd value at the sender is 2KB and the sender is in the slow start phase

- The maximum segment sixe of sender and receiver is 2KB.

- The receiver’s window size is 70KB

- The sender is always has some data to be sent to the receiver and ACKs are never dropped in the network.

Given the above conditions, after how many round trips (TRRs), will the sender successfully receive ACK for the last byte of the 402KB file that it has sent to the receiver. 								                           (4M)

c) Consider a sliding window protocol with a window size of 8 frames. If the round-trip time is 20 ms and the acknowledgment time is 2 ms, calculate the throughput (in frames per second) for the protocol.						               (4M)

d) Determine the optimal timeout value for a sliding window protocol given the following parameters:

Round-trip time = 30 ms

Estimated one-way propagation delay = 10 ms

Safety margin = 2									   (4M)

- a) Explain the concept of subnetting in IPv4 addressing and its role in optimizing network

resource utilization. 							               (5M)

b) Explain the relationship between ICMP, ARP, and DHCP in the context of network communication and management.					                           (7M)

c) Suppose that instead of using 16 bits for the network part of a class B address originally, 20 bits had been used. How many class B networks would there have been?          (5M)

d) A network on the Internet has a subnet mask of 255.255.240.0. What is the maximum number of hosts it can handle?					                           (3M)