I MCA II Semester Regular Examinations – May, 2025

COMPUTER NETWORKS

Time: 150 Min                                               (MCA)                                   Max. Marks:60M

SECTION – A

Answer all Four questions 							          4×8M=32M

1. a) A university wants to connect all its departments across a campus. Which type of network is suitable and why?						                     	         [2M]

b) Compare OSI and TCP/IP models focusing on their layer functionalities.                [3M]

c) Suppose a fibre-optic cable is used for long-distance transmission. Discuss its advantages and limitations over coaxial and twisted-pair cables.	                                             [3M]

2. a) List any two framing methods used in the data link layer.  			         [2M]

b) Given the data word 10011101 and generator 1001, compute the CRC code and final frame to be transmitted.						                                 [4M]

c) Analyze the efficiency of the Stop-and-Wait protocol when the transmission time is 10ms and round-trip delay is 50ms.					                                 [2M]

3. a) Discuss the role of Quality of Service (QoS) in supporting performance requirements of multimedia applications such as video conferencing.	                                             [2M]

b) A router receives the following routing table. Given a destination IP of 192.168.1.45, determine the output interface. Provide justification. 		                                 [3M]

| Destination | Subnet Mask | Interface |
| --- | --- | --- |
| 192.168.1.0 | 255.255.255.0 | Eth0 |
| 192.168.0.0 | 255.255.0.0 | Eth1 |
| 0.0.0.0 | 0.0.0.0 | Eth2 |

- Compare distance vector and link state routing in terms of convergence and scalability.

[3M]

4. a) Why is flow control essential in the transport layer?		                                 [2M]

b) Compare the header fields of UDP and TCP. In what cases is UDP preferred?         [3M]

c) A DNS resolver receives a query for www.vignan.ac.in Describe how the name resolution process proceeds with each DNS server involved.	                                              [3M]

SECTION – B

Answer all Two questions						       	        2×14M=28M

5.  a) Draw and explain a protocol stack diagram for Bluetooth, highlighting key layers. [3M]

b) Vignan’s University plans to upgrade its LAN to a metropolitan network. What physical   media and protocol architecture would you recommend? Justify.	                      [3M]

- A bit stream 011010101 is to be transmitted. Apply bit stuffing and show the result. 
    									                      [2M]

- Discuss how CSMA/CD handles collisions and why it is less effective in wireless media.									                                  [4M]

- Explain the advantages and disadvantages of using parity, CRC, and checksum for error detection. 								                      [2M]

6. a) A network has routers A-B-C-D-E-F-G with the following link costs. Apply Dijkstra’s algorithm to find the shortest path from A to F.

[5M]

- Explain the need for sub netting and calculate the number of subnets and hosts possible for the address 192.168.10.0/26.	 		                                              [3M]

- Compare TCP's congestion control phases with appropriate diagrams.                     [3M]

- Explain how email is delivered using SMTP, POP3, and IMAP protocols. 
 								                                              [3M]