COMPUTER NETWORKS – ANSWERS FOR 3 QUESTION PAPERS

DOCUMENT 1

24MC110-1 (May 2025)

SECTION – A

1(a) Suitable Network for University Campus

A Campus Area Network (CAN) is suitable because it connects multiple departments within a limited geographical area such as a university campus. It provides high-speed communication, centralized management, resource sharing, and secure data transfer.

1(b) OSI vs TCP/IP Models

| OSI Model | TCP/IP Model |
| --- | --- |
| 7 Layers | 4 Layers |
| Reference model | Practical model |
| Session and Presentation layers separate | Combined into Application layer |
| Developed by ISO | Developed by DARPA |
| Protocol independent | Protocol dependent |

1(c) Advantages and Limitations of Fiber Optic Cable

Advantages:

- High bandwidth

- Long distance communication

- Immune to electromagnetic interference

- Secure transmission

Limitations:

- Expensive installation

- Fragile cable

- Difficult maintenance

Compared to coaxial and twisted pair cables, fiber provides faster and more reliable communication.

2(a) Framing Methods

- Character Count Method

- Bit Stuffing Method

2(b) CRC Calculation

Data word = 10011101
Generator = 1001

After CRC division, remainder = 100

Final transmitted frame = 10011101100

2(c) Stop-and-Wait Efficiency

Efficiency = Transmission Time / (Transmission Time + Round Trip Time)

= 10 / (10 + 50)
= 10/60
= 0.166

Efficiency = 16.6%

3(a) Role of QoS

Quality of Service (QoS) ensures guaranteed bandwidth, reduced delay, controlled jitter, and minimum packet loss. It is important for multimedia applications like video conferencing and online streaming.

3(b) Output Interface Determination

Destination IP = 192.168.1.45

Matching:

- 192.168.1.0/24 → Eth0

- 192.168.0.0/16 → Eth1

Longest prefix match is selected.

Output Interface = Eth0

3(c) Distance Vector vs Link State Routing

| Distance Vector | Link State |
| --- | --- |
| Slow convergence | Fast convergence |
| Less scalable | Highly scalable |
| Uses Bellman-Ford | Uses Dijkstra |
| Shares routing tables | Shares topology information |

4(a) Need for Flow Control

Flow control prevents the sender from overwhelming the receiver with excessive data. It ensures reliable communication and avoids packet loss.

4(b) TCP vs UDP

| TCP | UDP |
| --- | --- |
| Connection-oriented | Connectionless |
| Reliable | Unreliable |
| Error checking | Minimal error checking |
| Large header | Small header |

UDP is preferred in video streaming, gaming, and VoIP where speed is important.

4(c) DNS Name Resolution

- Resolver sends request to local DNS server.

- Local DNS contacts Root DNS server.

- Root DNS points to .in server.

- .in server points to ac.in server.

- ac.in server points to vignan.ac.in server.

- Authoritative DNS returns IP address.

- Resolver sends IP to user.

SECTION – B

5(a) Bluetooth Protocol Stack

Layers:

- Radio Layer

- Baseband Layer

- LMP

- L2CAP

- RFCOMM

- Application Layer

Bluetooth supports short-range wireless communication.

5(b) Recommended Media and Protocol

Recommended Media:

- Fiber optic cable

Recommended Protocol:

- Metro Ethernet

Reason:

- High bandwidth

- Reliability

- Scalability

- Long distance communication

5(c) Bit Stuffing

Input = 011010101

Bit Stuffed Output = 011010101
(No sequence of five consecutive 1s exists.)

5(d) CSMA/CD

CSMA/CD senses the channel before transmission. If collision occurs, devices stop transmission and retransmit after random delay.

Wireless media is not suitable because collision detection is difficult due to hidden terminal problem.

5(e) Parity vs CRC vs Checksum

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Parity | Simple | Detects only single-bit errors |
| CRC | Highly accurate | Complex implementation |
| Checksum | Easy calculation | Less reliable than CRC |

6(a) Dijkstra Algorithm

Dijkstra algorithm finds the shortest path between source and destination nodes using minimum cost selection.

Shortest path from A to F is determined by continuously selecting the nearest node.

6(b) Subnetting

Address = 192.168.10.0/26

Subnet bits = 2
Number of subnets = 2² = 4

Host bits = 6
Hosts per subnet = 2⁶ – 2 = 62

6(c) TCP Congestion Control Phases

- Slow Start

- Congestion Avoidance

- Fast Retransmit

- Fast Recovery

These phases control network congestion and improve performance.

6(d) Email Delivery Process

SMTP:

- Sends emails

POP3:

- Downloads emails from server

IMAP:

- Synchronizes emails across devices

DOCUMENT 2

22MC110-1 (June 2023)

SECTION – A

1(a) Checksum Calculation

Data:

- 0x3456

- 0xABCC

- 0x02BC

- 0xEEEE

Checksum is calculated using one’s complement addition.

Receiver recalculates checksum. If mismatch occurs, error is detected.

Checksum cannot identify which specific data item is corrupted.

1(b) Average Frame Delay

Transmission rate = 50 Mbps
Frame length = 5000 bits

Transmission time:
= 5000 / 50×10⁶
= 0.1 ms

Queueing delay increases as frame arrival rate increases.

For 900 frames/sec, delay becomes significantly larger.

2 CRC Error Detection

CRC with divisor polynomial x⁵ + 1:

(a) Detects all odd number bit errors.

(b) Does not guarantee detection of all even number bit errors.

(c) Detects all burst errors of length less than or equal to degree of generator polynomial.

3 TCP Congestion Control

Initial window size = 2 MSS
Threshold = 8 MSS

Transmission sequence:

- 1st = 2 MSS

- 2nd = 4 MSS

- 3rd = 8 MSS

- 4th = 9 MSS

- Timeout during 5th transmission

After timeout:

- Threshold = half of previous cwnd

- cwnd reset to 1 MSS

Window size gradually increases again.

4 Communication Protocols

Communication protocol is a set of rules governing data communication between devices.

Importance:

- Ensures reliable communication

- Defines syntax and timing

- Enables interoperability

HTTP

Used for web communication.

SSH

Used for secure remote login.

SMS

Used for text messaging in mobile networks.

SECTION – B

5 Dijkstra Algorithm

Dijkstra algorithm calculates shortest path by selecting node with minimum distance repeatedly until all nodes are visited.

6 Distance Vector Routing

Distance vector routing uses hop count and routing tables to determine shortest path.

Nodes exchange routing information periodically with neighbors.

DOCUMENT 3

22MC110-3 (May 2024)

SECTION – A

1(a) OSI vs TCP/IP

OSI model has seven layers whereas TCP/IP has four layers.

OSI is theoretical and TCP/IP is practical.

TCP/IP combines session and presentation layers into application layer.

1(b) Connection-Oriented Transmission

Connection-oriented communication establishes a connection before data transfer.

Characteristics:

- Reliable

- Ordered delivery

- Error control

- Acknowledgments used

Connectionless transmission does not establish a connection.

1(c) Benefits of Connection-Oriented Transmission

Benefits:

- Reliable file transfer

- Error recovery

- Data sequencing

- Secure communication

Useful for transferring large files between company offices.

2(a) Objectives of Flow Control

Objectives:

- Prevent receiver overload

- Improve efficiency

- Avoid packet loss

- Maintain synchronization

Mechanisms:

- Stop-and-Wait

- Sliding Window

2(b) Parity Bit

Message = 1101101

Number of 1s = 5

For even parity:
Parity bit = 1

Transmitted message = 11011011

2(c) CRC Method

Generator polynomial = x³ + 1
Binary divisor = 1001

CRC bits are appended to message.

If third bit changes during transmission, remainder at receiver becomes non-zero and error is detected.

3(a) Design Issues and Routing Algorithms

Design Issues:

- Routing

- Congestion control

- Addressing

- Packet forwarding

Distance Vector:

- Uses routing tables

- Slow convergence

Link State:

- Uses network topology

- Fast convergence

3(b) Classful Addressing

Class A:

- Large networks

Class B:

- Medium networks

Class C:

- Small networks

Problems:

- Address wastage

- Poor scalability

3(c) Address Allocation

Organizations receive address blocks according to required number of hosts.

Subnet masks are assigned based on address requirements.

4(a) Email as Asynchronous Communication

Email is asynchronous because sender and receiver need not communicate at the same time.

Messages are stored temporarily on mail servers.

4(b) Email Protocols

SMTP:

- Sends email

POP3:

- Retrieves email

IMAP:

- Synchronizes email on multiple devices

4(c) HTTP Protocol

HTTP follows request-response model.

Features:

- Stateless protocol

- Uses methods like GET, POST, PUT, DELETE

GET:
Retrieve data

POST:
Submit data

PUT:
Update data

DELETE:
Delete resource

SECTION – B

5(a) Stop-and-Wait vs Sliding Window

| Stop-and-Wait | Sliding Window |
| --- | --- |
| Sends one frame at a time | Sends multiple frames |
| Lower efficiency | Higher efficiency |
| Simple | Complex |
| Slow transmission | Faster transmission |

5(b) TCP Congestion Problem

Sender gradually increases congestion window during slow start until threshold is reached.

Then congestion avoidance phase begins.

5(c) Sliding Window Throughput

Window size = 8
RTT = 20 ms
ACK time = 2 ms

Throughput = Window size / RTT
= 8 / 0.022
= 363 frames/sec approximately.

5(d) Timeout Value

Timeout = RTT + Safety Margin
= 30 + 2
= 32 ms

6(a) IPv4 Subnetting

Subnetting divides a large network into smaller networks.

Advantages:

- Better management

- Reduced congestion

- Efficient IP usage

6(b) ICMP, ARP, DHCP

ICMP:

- Error reporting and diagnostics

ARP:

- Converts IP address to MAC address

DHCP:

- Dynamically assigns IP addresses

6(c) Class B Networks

Original Class B uses 16 network bits.

If 20 bits are used:
Number of networks = 2²⁰
= 1,048,576 networks

6(d) Maximum Hosts

Subnet mask = 255.255.240.0

Host bits = 12

Maximum hosts = 2¹² – 2
= 4094 hosts